Abstract

Zinc oxide attracts worldwide interest due to its direct wide band gap of 3.3 eV (at 300 K) and large room temperature exciton binding energy of 60 meV, which allows it to be successfully used in UV-blue optoelectronic devices (e.g. LEDs or detectors) and makes it a serious competition for commonly used gallium nitride. The material is clear and colourless, exhibits high electron concentration and can be used as contact in transparent electronics. However it is a natural n-type semiconductor which makes achieving p-type conductivity in this material quite a challenge. Group V elements seem to be a good candidate for p-type dopants and many reports confirm this, although the origin of p-type conductivity in ZnO doped with group V is not entirely understood therefore the topic still requires more research.

Among all available epitaxial growth methods, Molecular Beam Epitaxy seems to be especially interesting due to its high purity and possibility to control every aspect of growth process. Employing MBE in the studies regarding doping allows then to minimize the influence of unintentional impurities and design in details the growth process focusing only on the dopants themselves without a concern about purity of investigated structures.

Raman spectroscopy on the other hand is a powerful tool that gives an insight on structural properties of investigated materials. The technique employs inelastic scattering of light on phonons in the material in order to provide information on its composition, internal stress, crystalline quality and contamination. The method is very sensitive to breaking of crystal lattice symmetry therefore it seems to be perfect for investigating doped materials since introducing dopants into crystal structure results in breaking its translational symmetry.

In this work a complex analysis of Raman scattering in zinc oxide grown with MBE and doped with group V elements was conducted. The investigated dopants were antimony, arsenic and nitrogen. Alloying ZnO with MgO was proven to influence incorporation of dopants into the crystal lattice of ZnO, therefore ZnMgO samples were also investigated in the dissertation. Samples were grown on two types of substrates: a-Al2O3 and r-Al2O3 which resulted in layers oriented in c- and a-direction, respectively. All samples were grown with the use of the same high-purity method, i.e. MBE which allowed for objective comparative analysis of obtained results. Investigated samples were designed to give a straightforward answer on the influence of doping, Mg incorporation and choice of substrate.

The research focused on two major directions: investigation of characteristics of ZnO E2 modes that were the basis for the strain calculations and analysis of disorder, and investigation of additional modes appearing in the spectra. The strain was calculated based on position of E2high mode and full width at half-maximum (FWHM) of both E2 modes gave an insight on disorder in the layers. The characteristics of additional modes were analysed in terms of incorporated dopant, magnesium addition and influence of post-growth annealing. The origin of additional modes was also discussed in details.

07.06.2021